# HOW TO BOOST THE FLOW SHOP MANUFACTURING AGILITY USING HYBRID GENETIC TABU SEARCH IN SCHEDULING

| Abstract The hybridization between evolutionary genetic algorithm and tabu search has been proposed in this paper to address flow shop scheduling. It accommodates jobs that need to be rearranged and executed on identical machines serially. High agility is required in the manufacturing process, especially for the garment industry to be able to stand facing competitors. The manufacturing related to scheduling to deliver a product as early as possible, the tardiness and waiting time also concerned. To deal with this, a genetic algorithm was widely used; which find an optimal solution to the problems because can obtain a more optimal solution. Unfortunately, it is easy to get stuck in optimum local (early convergence is faster). The tabu search algorithm works as a local explorer to find and exploit the optimum local area better and this can be combined with a generic algorithm. This study aims to minimize the three objectives mentioned above to increase production agility. To show the significance of the proposed algorithm, these strategies are evaluated on Taillard benchmark problems. The results prove that the hybrid mechanism capable to enhance the solution quality by 2.75% compared to our previous work and can resolve all of Taillard instance better, has been proven by 0.28% of percentage relative deviation which shows the error rate, smaller is better. This is an open access article under the CC BY-NC license | Keywords: Agility; Genetic Algorithm; Flow Shop; Production Scheduling Article History: Received: May 2, 2019 Revised: May 29, 2019 Accepted: June 2, 2019 Published: June 2, 2019 Corresponding Author: Andi Adriansyah Electrical Engineering Department, Universitas Mercu Buana, Indonesia Email: andi@mercubuana.ac.id |
| --- | --- |

INTRODUCTION

The growth of the fashion industry reached the international business level across all sectors – from independent boutiques to worldwide brands, from raw material to finished goods selling, and also comes from small retailers to big wholesalers [1]. The fashion industry business model is required to provide services or goods as quickly as possible to the shopper who are known for their agility due to the influence of information technology innovation, short product cycles under competitive market pressures, changing consumer needs, and unreliable demand conditions [2]. Agility is strongly influenced by activities in the production process, especially in the scheduling of products to be made [3], so that good production scheduling is needed in order to adapt to varying market changes. Scheduling is one of the most important elements in business administration and information systems because a good schedule allows management to have all necessary parts at hand when they are needed [4].

Production scheduling is defined as a process that combines sales forecasting and manufacturing planning [5]. They are important to control resources in a complex production environment with many different machines and jobs. While the two are separate, when combined, they have the potential to increase a business's productivity, accuracy, and agility [6]. A well-known scheduling problem that attracts a large number of researchers is flow shop scheduling which is used when the product is similar in nature and requires almost identical processing time for each job such as garment production [7]. Flow shop is a family of scheduling problems for which there is a precedence relationship between tasks and also capacity limits on resources along with precedence constraints [8]. Production scheduling is the means to control the resources (machines) to produce useful results at minimum cost with maximum effectiveness. Unfortunately, a problem occurs in scheduling when a number of tasks have to be completed in a given time with limited resources, but there is no clear instruction on which tasks should be completed first and which later.

Genetic algorithm (GA) is widely applied to deal with scheduling problems [9] categorized as an evolutionary algorithm that is motivated by genetic evolution through natural selection [10]. The algorithm selects the optimal solution for its fitness or objective function and puts it into a new generation. In some cases, those solutions are then modified by operators such as crossover, mutation or some other manipulating algorithms. The common sense of GA is that it copies the parent's strategy and modifies it for producing children, which are realistic solutions hard to be found through common optimization algorithms. Unfortunately, GA often suffers from the drawbacks of premature convergence and weak exploitation capabilities so that trapped in optimum local [11].

On other place, there is a good algorithm in performing exploitation for flow shop where the goal is to find a solution with high quality, rather than merely find a solution called with tabu search [12] [13]. The tabu search idea is quite straightforward. The algorithm begins with a solution and then searches its neighborhood for a suitable new neighborhood. By allowing moves to neighbors with poorer makespans, the search approach extends beyond local optimality to examine the solution space. The important aspects of the exploration path are selectively memorized (via a tabu list), then proactive decisions are taken so that it can steer the exploration away from optimum local toward different areas inside solution space. If a halting criterion is met then the algorithm terminates itself.

To address the GA's shortcomings, this study proposes a hybrid method. These are tabu search algorithms combined with GA to increase the manufacturing agility in the garment. The rest of the article's structure is mentioned as the following: A concise summary of the research that has been done is presented in the materials and method section which also discusses the fusion of the proposed GA and TS algorithms. Experimental studies are presented in the next section and conclusions with future works in the conclusion section.

METHOD

An improved hybridization between GA and TS method is proposed for achieving the manufacturing agility using three steps: first data gathered from the Taillard dataset is partitioned into a number of subsets. Second, GA is applied to the first subset in order to get the initial population. Third, TS is utilized to enhance the solution quality found by GA.

The Proposed Algorithms

Throughout this section, the multi-objective scenario is given first as the paper contribution, which is to develop an optimization algorithm based on partial opposed-based learning. Notably, the initialization uses a partial opposed-based approach, which is consistent with our previous work [14]. There are a few parallels between them. In detail, both of these algorithms randomize then divide the population into two sections and create one of them using an opposed-based strategy. According to our earlier work, this algorithm is only applicable to single-objective optimization problems. The number of objectives in this paper is significantly greater than in our prior work, it is designed to deal with multi-objective optimization problems. Thus, it is started by describing a multi-objective problem, followed by GA and TS parts.

1. Multi-Objective Problem

The purpose of this multi-objective optimizer for scheduling is to find a feasible solution under three different objectives: finding the shortest job completion time, minimal tardiness, and total waiting time between jobs at the same time. The optimal solution is declared as the following:

Min fa(x),fb(x),fc(x)		        (1)

Supplied with x∈X; where fa(x),fb(x),fc(x) are the objectives to be minimized, x supposed as decision vector, meanwhile, the X supposed as the decision space.

2. Inserting TS into GA

The key idea of the GA is based on the fact that human beings make decisions by comparing the current situation with the previous situation. In order to make GA more intelligent, TS helps GA by integrating TS into the GA process to result in a more optimal solution. It uses fine-tuned parameter configuration provided in Table 1 and contains some phases:

Table 1. Configuration for GA-TS

| No | Configuration | Size |
| --- | --- | --- |
| 1 | Iteration | 1000 |
| 2 | Population | 100 |
| 3 | Crossover | 0.5 |
| 4 | Mutation | 0.1 |
| 5 | Tournament length | 5 |
| 6 | Tabu length | 5 |

Phase 1: encoding the solution

- Gene  set of job

- Chromosome  set of job sequence

- Distribute time t needed on every gene

Phase 2: population initialization

- Generate n size population randomly

- Split population becomes two section

- Process section part by partial opposition based strategy to take the best fitness by comparing opposite point

Phase 3: fitness checking

- If iteration is not reached do

- Evaluate population by fitness objective function

- Update the solution

Phase 4: parent selection

- E  elitist fitness

- F  current fittest solution

- For individual to tournament size do

- NF  new fittest solution resulted from population

- If NF > F

- F  NF

- Return F and save to elitist

Phase 5: offspring generation

- // crossover (two-point)

- Randomly decide two chromosome

- Choose two barriers to exchange chromosome

- Exchange chromosome inside the barrier to generating solution

- // mutation (swap)

- Select two points in a chromosome

- Swap the chosen point to generate a new solution

Phase 6: tabu search task

- Rule the tabu list size, aspiration criteria, and stop regulation

- Do a move to search solution space using insertion and swap strategy

- Renew the solution using the best new one which is not saved in the tabu list

- Stop exploration when stop regulation fulfilled

Phase 7: decoding the solution

- Gathering job sequence

- Gathering machine with processing time

- Load Gantt chart construction

Figure 1. Proposed GA-TS Algorithm

Visually, the algorithm runs using flow as shown in Figure 1 started with solution encoding as sequence table followed by initializing the population using partial opposed based method from our earlier study. Then do a fitness checking, this paper has three objectives function to minimize the makespan or the shortest of job finished time, minimal tardiness, and total waiting time, respectively:

Min f1=C(Ji, m) 		        (2)

Min f2=i=1jj=2mT(Ji, m)	        (3)

Min f3=i=1jj=2mW(Ji, m)	        (4)

Where:

j 	  : set of job

m 	  : set of machine

C(Ji, m)	  : completion time job J on machine m

T(Ji, m)	  : tardiness for job J which processed

on machine m

W(Ji, m)  : waiting time job J, machine m

(i = 1, 2, …, n) and (j = 1, 2, …, m)

This study assumes the operation preemption is not allowed, thus, an operation of a job on a machine can be executed if only the previous operation already finished its process. Also, processing time may be zero because not all operations are processed on all machines, flow shop is only how to rearrange the order of the jobs using an identical machining sequence.

Next, two chromosomes are selected as a parent using tournament selection which is commonly used by the researcher [15] [16] [17], and the best chromosomes are saved to the elitist solution. Entering the reproduction phase 5, it is intended to generate offspring using two-point crossover [18] and swap mutation [19] to maintain diversity.

After passing the genetic operator, the chromosome is processed by tabu search exploiting the solution space using insertion and swap strategy [20] to result in the new solution. Finally, the algorithm decodes the optimal solution as the Gantt chart which provides job sequence and needed time to make products. Figure 2 shows the chart of how to schedule under three different jobs and types of machinery.

Figure 2. Example of Gantt Chart

RESULTS AND DISCUSSION

Throughout this section, the hybrid GA-TS is applied to the Taillard problem [21] to provide a fair computational experiment. This dataset is publicly available and contains the results of a competition to find the best possible solution to a problem. The dataset contains 120 instances, each of which has a different solution. All instances were treated by the compared algorithms for simplicity. By employing a 1.9 GHz processor, using memory 4 GB, and coded in Python language, the proposed algorithm is executed and the best solution is presented in Table 3 which summarizes the findings. Additionally, for statistical analysis, we collect the percentage relative deviation (PRD) of each of 120 instances over ten number run to show average error among a solution of the proposed algorithm and the lowest known upper bound values, where a lower PRD indicates a better algorithm, formulated as:

PRD= i=1N(GATS2-UBUB x 100)N  		         (5)

Where:

N	: number of instances

GATS2	: best solution from proposed GA-TS

UB	: Taillard upper bound, the best known

solution for Taillard

Regarding the hybrid strategy, GA has been hybridized to achieve one or more objectives with some various algorithms, such as tabu search [14] which successfully improve the solution quality by 115 of 120 Taillard instances over hybrid genetic simulated annealing and five other GA cooperation with PRD 3.05%. Thus, this proposed algorithm (GATS2) is compared to the algorithm on our previous work (GATS1), genetic algorithm variable neighborhood search (GAVNS) by [22] which is superior to single variable neighborhood search algorithm, and finally compared to hybrid evolution strategy (HESSA) studied by [23]. The proposed GA-TS solves each Taillard instance. To facilitate comparison, the best-known upper bounds or optimal solutions for these problems are utilized. We compare algorithm performance using the percentage of increase (PI) between the current algorithm upper bound and Taillard upper bound, calculated as:

PI= GATS2-UBUBx100 			        (6)

Here we go for the experiment result, please note that the most right column indicates the percentage of increase for GATS2 compared to every Taillard upper bound, this can be used to calculate PRD.

Table 3. Experimental Results

| Problem | UB | GAVNS | GATS1 | HESSA | GATS2 | PI |
| --- | --- | --- | --- | --- | --- | --- |
| Tai001 | 1278 | 1486 | 1282 | 1278 | 1278 | 0,00 |
| Tai002 | 1359 | 1528 | 1373 | 1359 | 1359 | 0,00 |
| Tai003 | 1081 | 1460 | 1098 | 1081 | 1081 | 0,00 |
| Tai004 | 1293 | 1588 | 1310 | 1293 | 1293 | 0,00 |
| Tai005 | 1235 | 1449 | 1277 | 1235 | 1235 | 0,00 |
| Tai006 | 1195 | 1481 | 1224 | 1195 | 1195 | 0,00 |
| Tai007 | 1239 | 1483 | 1251 | 1239 | 1239 | 0,00 |
| Tai008 | 1206 | 1482 | 1229 | 1206 | 1206 | 0,00 |
| Tai009 | 1230 | 1469 | 1257 | 1230 | 1230 | 0,00 |
| Tai010 | 1108 | 1377 | 1140 | 1108 | 1108 | 0,00 |
| Tai011 | 1582 | 2044 | 1622 | 1582 | 1582 | 0,00 |
| Tai012 | 1659 | 2166 | 1706 | 1659 | 1659 | 0,00 |
| Tai013 | 1496 | 1940 | 1555 | 1496 | 1496 | 0,00 |
| Tai014 | 1377 | 1811 | 1407 | 1377 | 1377 | 0,00 |
| Tai015 | 1419 | 1933 | 1481 | 1419 | 1419 | 0,00 |
| Tai016 | 1397 | 1892 | 1440 | 1397 | 1397 | 0,00 |
| Tai017 | 1484 | 1963 | 1556 | 1484 | 1484 | 0,00 |
| Tai018 | 1538 | 2057 | 1584 | 1538 | 1538 | 0,00 |
| Tai019 | 1593 | 1973 | 1616 | 1593 | 1593 | 0,00 |
| Tai020 | 1591 | 2051 | 1646 | 1591 | 1591 | 0,00 |
| Tai021 | 2297 | 2973 | 2331 | 2297 | 2297 | 0,00 |
| Tai022 | 2099 | 2852 | 2169 | 2099 | 2099 | 0,00 |
| Tai023 | 2326 | 3013 | 2389 | 2326 | 2326 | 0,00 |
| Tai024 | 2223 | 3001 | 2306 | 2223 | 2223 | 0,00 |
| Tai025 | 2291 | 3003 | 2361 | 2291 | 2291 | 0,00 |
| Tai026 | 2226 | 2998 | 2297 | 2226 | 2226 | 0,00 |
| Tai027 | 2273 | 3052 | 2337 | 2273 | 2273 | 0,00 |
| Tai028 | 2200 | 2839 | 2249 | 2200 | 2200 | 0,00 |
| Tai029 | 2237 | 3009 | 2303 | 2237 | 2237 | 0,00 |
| Tai030 | 2178 | 2979 | 2287 | 2178 | 2178 | 0,00 |
| Tai031 | 2724 | 3161 | 2730 | 2724 | 2724 | 0,00 |
| Tai032 | 2834 | 3432 | 2890 | 2836 | 2834 | 0,00 |
| Tai033 | 2621 | 3211 | 2622 | 2621 | 2621 | 0,00 |
| Tai034 | 2751 | 3339 | 2780 | 2751 | 2751 | 0,00 |
| Tai035 | 2863 | 3356 | 2904 | 2863 | 2863 | 0,00 |
| Tai036 | 2829 | 3347 | 2867 | 2829 | 2829 | 0,00 |
| Tai037 | 2725 | 3231 | 2755 | 2725 | 2725 | 0,00 |
| Tai038 | 2683 | 3235 | 2701 | 2686 | 2683 | 0,00 |
| Tai039 | 2552 | 3072 | 2601 | 2552 | 2552 | 0,00 |
| Tai040 | 2782 | 3317 | 2783 | 2782 | 2782 | 0,00 |
| Tai041 | 2991 | 4274 | 3100 | 3024 | 3024 | 1,10 |
| Tai042 | 2867 | 4177 | 3017 | 2882 | 2882 | 0,52 |
| Tai043 | 2839 | 4099 | 3015 | 2852 | 2852 | 0,46 |
| Tai044 | 3063 | 4399 | 3124 | 3063 | 3063 | 0,00 |
| Tai045 | 2976 | 4322 | 3123 | 2982 | 2982 | 0,20 |
| Tai046 | 3006 | 4289 | 3188 | 3006 | 3006 | 0,00 |
| Tai047 | 3093 | 4420 | 3226 | 3122 | 3099 | 0,19 |
| Tai048 | 3037 | 4318 | 3207 | 3042 | 3038 | 0,03 |
| Tai049 | 2897 | 4155 | 3045 | 2911 | 2902 | 0,17 |
| Tai050 | 3065 | 4283 | 3233 | 3077 | 3077 | 0,39 |
| Tai051 | 3850 | 6129 | 4064 | 3889 | 3889 | 1,01 |
| Tai052 | 3704 | 5725 | 3910 | 3714 | 3720 | 0,43 |
| Tai053 | 3640 | 5862 | 3875 | 3667 | 3667 | 0,74 |
| Tai054 | 3720 | 5788 | 3904 | 3754 | 3754 | 0,91 |
| Tai055 | 3610 | 5886 | 3929 | 3644 | 3644 | 0,94 |
| Tai056 | 3681 | 5863 | 3967 | 3708 | 3708 | 0,73 |
| Tai057 | 3704 | 5962 | 3968 | 3754 | 3754 | 1,35 |
| Tai058 | 3691 | 5926 | 3996 | 3711 | 3711 | 0,54 |
| Tai059 | 3743 | 5876 | 4064 | 3772 | 3772 | 0,77 |
| Tai060 | 3756 | 5958 | 3954 | 3778 | 3778 | 0,59 |
| Tai061 | 5493 | 6402 | 5502 | 5493 | 5493 | 0,00 |
| Tai062 | 5268 | 6240 | 5301 | 5268 | 5268 | 0,00 |
| Tai063 | 5175 | 6133 | 5213 | 5175 | 5175 | 0,00 |
| Tai064 | 5014 | 6025 | 5041 | 5014 | 5014 | 0,00 |
| Tai065 | 5250 | 6198 | 5323 | 5250 | 5250 | 0,00 |
| Tai066 | 5135 | 6087 | 5171 | 5135 | 5135 | 0,00 |
| Tai067 | 5246 | 6255 | 5320 | 5246 | 5246 | 0,00 |
| Tai068 | 5094 | 6130 | 5127 | 5094 | 5094 | 0,00 |
| Tai069 | 5448 | 6381 | 5506 | 5448 | 5448 | 0,00 |
| Tai070 | 5322 | 6384 | 5386 | 5322 | 5322 | 0,00 |
| Tai071 | 5770 | 8079 | 5962 | 5776 | 5770 | 0,00 |
| Tai072 | 5349 | 7886 | 5594 | 5360 | 5349 | 0,00 |
| Tai073 | 5676 | 8028 | 5790 | 5677 | 5677 | 0,02 |
| Tai074 | 5781 | 8348 | 5939 | 5792 | 5781 | 0,00 |
| Tai075 | 5467 | 7958 | 5637 | 5467 | 5467 | 0,00 |
| Tai076 | 5303 | 7814 | 5401 | 5311 | 5304 | 0,02 |
| Tai077 | 5595 | 7866 | 5667 | 5596 | 5596 | 0,02 |
| Tai078 | 5617 | 7913 | 5633 | 5625 | 5625 | 0,14 |
| Tai079 | 5871 | 8166 | 5926 | 5891 | 5875 | 0,07 |
| Tai080 | 5845 | 8117 | 5867 | 5845 | 5845 | 0,00 |
| Tai081 | 6202 | 10700 | 6337 | 6257 | 6257 | 0,89 |
| Tai082 | 6183 | 10594 | 6403 | 6223 | 6223 | 0,65 |
| Tai083 | 6271 | 10611 | 6509 | 6342 | 6325 | 0,86 |
| Tai084 | 6269 | 10607 | 6409 | 6303 | 6303 | 0,54 |
| Tai085 | 6314 | 10539 | 6425 | 6380 | 6380 | 1,05 |
| Tai086 | 6364 | 10677 | 6419 | 6427 | 6431 | 1,05 |
| Tai087 | 6268 | 10835 | 6428 | 6306 | 6306 | 0,61 |
| Tai088 | 6401 | 10840 | 6540 | 6472 | 6472 | 1,11 |
| Tai089 | 6275 | 10723 | 6611 | 6380 | 6330 | 0,88 |
| Tai090 | 6434 | 10798 | 6514 | 6485 | 6456 | 0,34 |
| Tai091 | 10862 | 15319 | 11103 | 10872 | 10872 | 0,09 |
| Tai092 | 10480 | 15126 | 10637 | 10487 | 10487 | 0,07 |
| Tai093 | 10922 | 15398 | 11220 | 10941 | 10922 | 0,00 |
| Tai094 | 10889 | 15240 | 11075 | 10889 | 10889 | 0,00 |
| Tai095 | 10524 | 15259 | 10756 | 10524 | 10526 | 0,02 |
| Tai096 | 10326 | 15116 | 10465 | 10346 | 10330 | 0,04 |
| Tai097 | 10854 | 15415 | 11174 | 10868 | 10868 | 0,13 |
| Tai098 | 10730 | 15279 | 11002 | 10741 | 10731 | 0,01 |
| Tai099 | 10438 | 15135 | 10721 | 10451 | 10454 | 0,15 |
| Tai100 | 10657 | 15340 | 10785 | 10680 | 10680 | 0,22 |
| Tai101 | 11195 | 19740 | 11528 | 11287 | 11280 | 0,76 |
| Tai102 | 11203 | 20112 | 11650 | 11277 | 11272 | 0,62 |
| Tai103 | 11281 | 19937 | 12163 | 11418 | 11378 | 0,86 |
| Tai104 | 11275 | 19961 | 12098 | 11376 | 11376 | 0,90 |
| Tai105 | 11259 | 19849 | 11979 | 11365 | 11310 | 0,45 |
| Tai106 | 11176 | 19942 | 11651 | 11330 | 11265 | 0,80 |
| Tai107 | 11360 | 20112 | 11957 | 11398 | 11430 | 0,62 |
| Tai108 | 11334 | 20064 | 11716 | 11433 | 11398 | 0,56 |
| Tai109 | 11192 | 19918 | 12120 | 11356 | 11265 | 0,65 |
| Tai110 | 11288 | 19942 | 12004 | 11446 | 11355 | 0,59 |
| Tai111 | 26059 | - | 26771 | 26187 | 26187 | 0,49 |
| Tai112 | 26520 | - | 27014 | 26799 | 26779 | 0,98 |
| Tai113 | 26371 | - | 27491 | 26496 | 26494 | 0,47 |
| Tai114 | 26456 | - | 26902 | 26612 | 26618 | 0,61 |
| Tai115 | 26334 | - | 26790 | 26514 | 26500 | 0,63 |
| Tai116 | 26477 | - | 27297 | 26661 | 26647 | 0,64 |
| Tai117 | 26389 | - | 26758 | 26529 | 26529 | 0,53 |
| Tai118 | 26560 | - | 27134 | 26750 | 26772 | 0,80 |
| Tai119 | 26005 | - | 27636 | 26223 | 26223 | 0,84 |
| Tai120 | 26457 | - | 27049 | 26619 | 26617 | 0,60 |

Table 3 demonstrates that the GA-TS method makes deeper exploitation inside the solution space and discovers a higher quality of solutions. Visual charts of the data can be seen through Figure 3 for instance 1 to 60, and Figure 4 for instance 61 to 120. From these two figures, we know that HESSA and GATS2 are very close in the resulting solution, so we bold the better (the smaller) solution between the two algorithms in Table 3.

Figure 3. Solution for Tai001 – Tai060

Figure 4. Solution for Tai061 – Tai120

From the computation time used, we can average the time needed in Table 4. The table shows that the bigger problem instance to be solved, it needs more time for the proposed GA-TS to solve.

Table 4. Average Execution Time of GA-TS

| Problem | Time (s) | Problem | Time (s) |
| --- | --- | --- | --- |
| 20 ~ 5 | 0,994 | 100 ~ 5 | 9,797 |
| 20 ~ 10 | 2,337 | 100 ~ 10 | 27,335 |
| 20 ~ 20 | 5,872 | 100 ~ 20 | 65,923 |
| 50 ~ 5 | 3,671 | 200 ~ 10 | 76,429 |
| 50 ~ 10 | 13,138 | 200 ~ 20 | 246,892 |
| 50 ~ 20 | 34,058 | 500 ~ 20 | 839,868 |

Based on Table 3, we can calculate the PI column, the sum is 33.46. So the PRD for the GA-TS algorithm calculated as follow:

PRD= 33.46120=0.28%

Comparison of PRD for another algorithm presented as Table 5.

Table 5. PRD Comparison

| Algorithm | Instance | PI | PRD (%) |
| --- | --- | --- | --- |
| GAVNS | 110 | 4588,16 | 41,71 |
| GATS1 | 120 | 366,09 | 3,05 |
| HESSA | 120 | 40,67 | 0,34 |
| GATS2-120 | 120 | 33,46 | 0,28 |
| GATS2-110 | 110 | 26,87 | 0,24 |

There is a difference between GAVNS, it is only used the first 110 of Taillard instance, so we calculate for the GATS2 twice to make a fair comparison. The first contains 120 instances, and the second contains the first 110 instances. The result that both GA-TS using 110 or 120 instances achieve the lowest PRD among others which is the best, visually can be conferred as Figure 5.

Figure 5. PRD Comparison

The main goal of this hybridization is to combine the power and flexibility of both algorithms in such a way that each algorithm contributes to the solution process in an effort to solve the objective function. Any problem that is solved using both GA and TS can benefit from this algorithm since its hybridization feature brings significant improvements in solution quality. We can measure the improvement using a percentage of increase provided in Table 6 which shows the comparison of proposed GA-TS (GATS2) algorithm improvement from other algorithms.

Table 6. Percentage of Increase

| Problem Size | PI of GATS2 Compared to |  |  |
| --- | --- | --- | --- |
|  | GAVNS | GATS1 | HESSA |
| 20 ~ 5 | 21,10 | 1,78 | 0,00 |
| 20 ~ 10 | 31,01 | 3,15 | 0,00 |
| 20 ~ 20 | 32,97 | 3,04 | 0,00 |
| 50 ~ 5 | 19,50 | 0,98 | 0,02 |
| 50 ~ 10 | 42,81 | 4,52 | 0,12 |
| 50 ~ 20 | 57,70 | 5,97 | -0,02 |
| 100 ~ 5 | 18,67 | 0,85 | 0,00 |
| 100 ~ 10 | 42,43 | 2,00 | 0,09 |
| 100 ~ 20 | 68,43 | 1,75 | 0,14 |
| 200 ~ 10 | 42,96 | 2,04 | 0,04 |
| 200 ~ 20 | 76,10 | 4,89 | 0,32 |
| 500 ~ 20 | - | 2,06 | 0,01 |
| Average | 41,24 | 2,75 | 0,06 |

The GA-TS algorithm is able to improve solution quality in all of Taillard instance, Figure 6 show the percentage of achieved improvement. The GA-TS was able to improve HESSA by 0.06% average, improve the GAVNS by 41.24% average, and improve our previous work on GA-TS by 2.75% average.

Figure 6. PI of GA-TS Compared

CONCLUSION

This paper gave GA-TS hybridization to deal with the scheduling problem of flow shop in terms of improving solution quality. By using our partial opposed initialization in GA, it is effective in addressing multi-objective scenarios. In addition, proper parameter tuning such as two-point crossover and simple swap mutation help GA-TS to perform their task easily. By employing the insertion and swap method for TS the GA perform exploitation deeply and results in better solution quality. From conducted experiment shows that the GA-TS have the lowest PRD (0.28%) and can improve all of the Taillard instances. Also can improve our previous work by 2.75%.

The future work is to examine the extensive use of another evolutionary algorithm combined with GA because of its superiority.

REFERENCES

[1]	B. Ly, “Competitive advantage and internationalization of a circular economy model in apparel multinationals,” Cogent Bus. Manag., vol. 8, no. 1, p. 1944012, Jan. 2021, doi: 10.1080/23311975.2021.1944012.

[2]	G. Gonda, E. Gorgenyi-Hegyes, R. J. Nathan, and M. Fekete-Farkas, “Competitive factors of fashion retail sector with special focus on SMEs,” Economies, vol. 8, no. 4, p. 95, Nov. 2020, doi: 10.3390/economies8040095.

[3]	E. S. A. Nasr, A. Afify, M. Osman, and S. A. Elatty, “Scheduling flow shop manufacturing systems considering agility issues,” Int. J. Collab. Enterp., vol. 4, no. 3, p. 205, 2014, doi: 10.1504/ijcent.2014.066331.

[4]	P. Jonsson and L. Kjellsdotter Ivert, “Improving performance with sophisticated master production scheduling,” Int. J. Prod. Econ., vol. 168, pp. 118–130, Oct. 2015, doi: 10.1016/j.ijpe.2015.06.012.

[5]	S. Zheng, J. Gao, and J. Xu, “Research on production planning and scheduling based on improved collaborative optimization,” Concurr. Eng. Res. Appl., vol. 27, no. 2, pp. 99–111, Jun. 2019, doi: 10.1177/1063293X19842253.

[6]	Budianto, Surachman, D. Hadiwidjojo, and Rofiaty, “The effect of manufacturing agility competencies on lean manufacturing in increasing operational performance,” Uncertain Supply Chain Manag., vol. 9, no. 1, pp. 195–204, 2021, doi: 10.5267/j.uscm.2020.10.001.

[7]	W. K. Wong, C. K. Chan, and W. H. Ip, “A hybrid flowshop scheduling model for apparel manufacture,” Int. J. Cloth. Sci. Technol., vol. 13, no. 2, pp. 115–131, 2001, doi: 10.1108/09556220110390737.

[8]	A. Goli, E. Babaee Tirkolaee, and M. Soltani, “A robust just-in-time flow shop scheduling problem with outsourcing option on subcontractors,” Prod. Manuf. Res., vol. 7, no. 1, pp. 294–315, Jan. 2019, doi: 10.1080/21693277.2019.1620651.

[9]	F. Werner, “A survey of genetic algorithms for shop scheduling problems,” in Mathematical Research Summaries, 2017, vol. 2, p. 15.

[10]	F. A. Omara and M. M. Arafa, “Genetic algorithms for task scheduling problem,” in Studies in Computational Intelligence, vol. 203, 2009, pp. 479–507.

[11]	I. Vlašić, M. Ðurasević, and D. Jakobović, “Improving genetic algorithm performance by population initialisation with dispatching rules,” Comput. Ind. Eng., vol. 137, no. 1, pp. 1–38, Nov. 2019, doi: 10.1016/j.cie.2019.106030.

[12]	V. A. Armentano and J. E. C. Arroyo, “An application of a multi-objective tabu search algorithm to a bicriteria flowshop problem,” J. Heuristics, vol. 10, no. 5, pp. 463–481, 2004, doi: 10.1023/B:HEUR.0000045320.79875.e3.

[13]	B. Ekşioǧlu, S. D. Ekşioǧlu, and P. Jain, “A tabu search algorithm for the flowshop scheduling problem with changing neighborhoods,” Comput. Ind. Eng., vol. 54, no. 1, pp. 1–11, Feb. 2008, doi: 10.1016/j.cie.2007.04.004.

[14]	M. S. Umam, M. Mustafid, and S. Suryono, “A hybrid genetic algorithm and tabu search for minimizing makespan in flow shop scheduling problem,” J. King Saud Univ. - Comput. Inf. Sci., Sep. 2021, doi: 10.1016/j.jksuci.2021.08.025.

[15]	E. C. Osuna and D. Sudholt, “Runtime analysis of restricted tournament selection for bimodal optimisation,” Evol. Comput., pp. 1–26, 2021, doi: 10.1162/evco_a_00292.

[16]	X. Han, L. Zheng, L. Wang, H. Zheng, and X. Wang, “Fireworks algorithm based on dynamic search and tournament selection,” Int. J. Comput. Appl., vol. 43, no. 6, pp. 577–588, 2021, doi: 10.1080/1206212X.2019.1590034.

[17]	A. Shukla, H. M. Pandey, and D. Mehrotra, “Comparative review of selection techniques in genetic algorithm,” 2015 1st Int. Conf. Futur. Trends Comput. Anal. Knowl. Manag. ABLAZE 2015, pp. 515–519, 2015, doi: 10.1109/ABLAZE.2015.7154916.

[18]	B. Kiraz, A. A. Bidgoli, H. Ebrahimpour-Komleh, and S. Rahnamayan, “A novel collective crossover operator for genetic algorithms,” IEEE Trans. Syst. Man, Cybern. Syst., vol. 2020-Octob, pp. 4204–4209, 2020, doi: 10.1109/SMC42975.2020.9282841.

[19]	K. W. Kim, M. Gen, and G. Yamazaki, “Hybrid genetic algorithm with fuzzy logic for resource-constrained project scheduling,” Appl. Soft Comput. J., vol. 2, no. 3, pp. 174–188, 2003, doi: 10.1016/S1568-4946(02)00065-0.

[20]	S. J. Beaty, “Genetic algorithms versus tabu search for instruction scheduling,” Artif. Neural Nets Genet. Algorithms, pp. 496–501, 1993, doi: 10.1007/978-3-7091-7533-0_72.

[21]	E. Taillard, “Benchmarks for basic scheduling problems,” Eur. J. Oper. Res., vol. 64, no. 2, pp. 278–285, 1993, doi: 10.1016/0377-2217(93)90182-M.

[22]	B. Jarboui, M. Eddaly, and P. Siarry, “A hybrid genetic algorithm for solving no-wait flowshop scheduling problems,” Int. J. Adv. Manuf. Technol., vol. 54, no. 9–12, pp. 1129–1143, 2011, doi: 10.1007/s00170-010-3009-4.

[23]	B. Khurshid, S. Maqsood, M. Omair, B. Sarkar, I. Ahmad, and K. Muhammad, “An improved evolution strategy hybridization with simulated annealing for permutation flow shop scheduling problems,” IEEE Access, vol. 9, pp. 94505–94522, 2021, doi: 10.1109/ACCESS.2021.3093336.