| # Effect of Diesel Particulate Filter (DPF) on Exhaust Gas Opacity, Smoke Temperature and Noise Level in 1997 Isuzu Panther Diesel Engine |  |
| --- | --- |

Agung Samudra1*, Muhammad Hudan Rahmat1, Sri Murwantini1, Thathit Suprayogi2

1Mechanical Engineering Education Study Program, Faculty of Teacher Training and Education, Universitas Palangka Raya, Indonesia

2Physics Study Program, Faculty of Mathematics and Natural Sciences, Universitas Palangka Raya, Indonesia

| Abstract This study aims to determine the effect of Diesel Particulate Filter (DPF) in reducing exhaust gas opacity, smoke temperature and noise level of 1997 Isuzu Panther. The DPF used in this study is made of copper and stainlees steel with metallic honeycomb model. This experimental study involved installing a DPF on the standard exhaust system of a diesel vehicle and measuring exhaust emissions, smoke temperature and noise levels. Tests were conducted under various conditions which include testing the standard exhaust (Non-DPF), standard exhaust with DPF (DPF), DPF with 50 grams of stainless steel (DPF + SS50), DPF with 100 grams of stainless steel (DPF + SS100), and DPF with 150 grams of stainless steel (DPF + SS150). The test results show that DPF significantly affects the reduction of exhaust opacity, smoke temperature and noise level of 1997 Isuzu Panther. The largest reduction in exhaust opacity, smoke temperature and noise level is produced by DPF + 150 grams of stainless steel with a reducible exhaust opacity of 87.1 ± 1.1%, smoke temperature of 43.2 ± 1.7%, and noise level of 15.2 ± 1.3%. This is an open access article under the CC BY-SA license | Keywords: DPF; Metallic Honeycomb; Copper; Stainless Steel; exhaust emissions Article History: Received: May 2, 2019 Revised: May 29, 2019 Accepted: June 2, 2019 Published: June 2, 2019 Corresponding Author: Agung Samudra Mechanical Engineering Education Study Program, Universitas Palangka Raya, Indonesia Email: agungsamudra@fkip.upr.ac.id |
| --- | --- |

INTRODUCTION

The development of transportation in Indonesia is increasing every year, this can be seen from the latest data from the Indonesia statistics that the number of motorized vehicles in Indonesia in 2023 has increased [1]. The Indonesian Police Traffic Corps in 2024 also stated that the data on the number of vehicles had reached 164,136,793 units [2]. The increasing number of vehicles on the highway of course also causes quite serious problems, especially the problem of air pollution [3]. People who live near the highway are more at risk of diseases due to air pollution [4]. In addition, vehicular pollution also poses a risk to the health of students who attend schools near the highway [5]. Diesel cars are one type of vehicle that produces air pollution in the form of black smoke/soot [6], [7]. Soot is an air pollution particle formed from incomplete combustion in motor vehicles [8]. Soot (PM 2.5) is very harmful to human health because it can settle in the lungs and cause serious diseases such as lung cancer [9], can even cause disability and death [10], [11]. With this problem, a technology is needed that can overcome the problem of vehicle exhaust emissions, especially diesel engines. The technology used to reduce exhaust emissions is the Diesel Perticulate Filter (DPF) which functions as a filter or soot trap, so that the residual combustion gas coming out of the exhaust becomes clean and does not endanger health [9], [12], [13]

Zhang, et al. conducted a study that examined the effectiveness of using DPF in reducing diesel engine exhaust emissions. The results of this study concluded that DPF is considered an effective method for controlling particulate matter (PM) emissions from diesel engines and complies with national/regional laws and regulations such as CHINA VI, Euro VI, and EPA Tier3 [12]. This shows that the DPF can effectively reduce PM emissions from diesel engines, thus complying with the emission standards adopted in China. Similar research was also conducted by Phyo, et al. in their research examined the combustion and exhaust behavior of a 3 L four-cylinder common rail diesel engine with three different types of B7 conventional diesel fuel with and without a platinum diesel oxidation catalyst (DOC) system and a non-catalytic partial flow diesel particulate filter (P-DPF). The results concluded that diesel engine particulate emissions could be reduced by about 50% by using the P-DPF system [14]. These results show that DPFs are effective in reducing particulate emissions. However, because the material used is platinum, it requires more cost because the price of platinum is relatively expensive.

Figure 1. DPF design (a) DPF Casing Dimensians, (b) Casing and DPF, (c) DPF Inside the Case, (d) Complete DPF

Figure 1. DPF design (a) DPF Casing Dimensians, (b) Casing and DPF, (c) DPF Inside the Case, (d) Complete DPF

Research conducted by Bakhchin et al, on PM control technology concluded that further investigation of PM control technology is needed. This study found that the material used for SCR is platinum. They also suggested developing the technology in order to reduce the level of diesel engine particulate emissions with other cheaper materials [15]. This research is the same as research by Phyo et. al, the material used for DPF is platinum which is relatively expensive. Therefore, materials are needed that are cheaper but can be used effectively in reducing diesel engine exhaust emissions.

The DPF used in this research has a different size and material from previous research. This study aims to determine the effect of DPF made from copper and stainless steel with a honeycomb model with a mesh size of 5 mm in reducing exhaust opacity, smoke temperature and noise level of Isuzu Panther 1997.

METHOD

This research is experimental research [16]. This research will test exhaust gas opacity, smoke temperature and 1997 Isuzu Panther noise using standard exhaust and experimental exhaust (DPF). The research procedures include design, manufacture, and testing. Copper and stainless steel (SS) are used in this study as materials for making DPFs. Copper is a metal that can be used as a DPF material [17]. In addition, stainless steel (SS) is also a metal that can be used as a DPF material [18].

Designing

At the design stage, an important step that must be taken is to understand the needs and technical specifications of the DPF according to applicable emission regulations, such as Euro 5 or Euro 6. The DPF design can capture fine particles produced from diesel combustion without significantly reducing engine performance. This involves calculating the pore size, wall thickness,

Figure 2. DPF position on the 1997 Isuzu Panther car exhaust

Figure 2. DPF position on the 1997 Isuzu Panther car exhaust

and geometric shape of the filter. In addition, it is necessary in the design of the DPF, the material used is also very important. This material must be able to withstand high temperatures and provide sufficient durability so that it does not need to be replaced frequently. in this study, using a honeycomb DPF design made from copper and stainless steel. The honeycomb model is a model commonly used in catalysts, so this model can also be used in DPFs [19]. The DPF design is shown in Figure 1. The DPF will be installed on the exhaust of the 1997 Isuzu Panther (Figure 2).

Manufacturing

Figure 3. DPF

Figure 4. DPF Testing Flow (a) Isuzu Panther 1997, (b) Probe, and (c) Smoke Opacitymeter

(d) Sound Level Meter

Figure 3. DPF

Figure 4. DPF Testing Flow (a) Isuzu Panther 1997, (b) Probe, and (c) Smoke Opacitymeter

(d) Sound Level Meter

The DPF manufacturing process includes 2 stages, namely casing and DPF manufacturing. Making the casing is done according to the size of the DPF that will be inserted in it. The casing is made of iron plate with a thickness of 1 mm. The size of the inner diameter of the casing must match the size of the outer diameter of the DPF, so that the DPF can be inserted into it with precision and there should be no exhaust gas leakage when testing. Furthermore, the DPF manufacturing process uses copper material with a thickness of 1 mm and stainless steel in the form of fibers. The DPF is made according to the previously designed size. The DPF is made with a honeycomb model, where this model is used by researchers who have done in previous studies. The results of making the DPF can be seen in Figure 3.

Testing

The testing process was carried out to determine the results of the influence of the DPF on exhaust gas opacity, smoke temperature and noise level of the 1997 Isuzu Panther. Figure 5 is the flow of DPF testing carried out in this research.

Figure 4 shows the flow of testing/collecting data on the exhaust gas opacity and smoke temperature of a 1997 Isuzu Panther, which is carried out in several procedures. The first step was to prepare the Isuzu Panther car and smoke opacity meter and ensure that the car and test equipment were functioning properly and that there were no leaks in the exhaust pipe.

In addition, the test device must be able to measure opacity concentration (smoke density) at free running acceleration [20] and the test device must have a valid calibration certification [21]. Next, the engine was operated until it reached working temperature. The exhaust system is cleaned by stepping on the gas pedal or accelerating three times to the maximum engine speed. This step also serves to adjust the foot to the condition of the gas pedal. After that, the engine is left at idle for approximately five seconds. The gas sensor/probe was then inserted into the exhaust gas pipe to a depth of at least 30 cm to prevent data errors. Next, accelerate quickly but gently, according to the “accelerate” command on the smoke opacity meter monitor screen, and maintain it for 4 seconds until the “release/decelerate” command appears. After that, the gas pedal is released until the engine speed returns to idle, according to the SAE-J1667 test procedure (snap acceleration test procedure) [22]. The test result data was then printed or recorded in the data recording form. This test was conducted three times to obtain average results.

Figure 4 also shows the noise level data collection process and was conducted based on the ISO/FDIS 5130:2006 (E) [23] standard with the following test procedure: The first step is to prepare a noise level meter. Then, install the device at 0.5 meters from the axis point of the exhaust pipe at an angle of 45 degrees, and make sure the height of the device is not less than 0.2 meters from the ground. Ensure that there are no other sounds that can interfere with the testing process. Next, press the gas pedal to maximum speed as quickly as possible until the engine reaches maximum speed, then hold it for 3 to 5 seconds. After that, record the test results. Perform the test three times and calculate the average of the results.

RESULTS

Smoke Opacity

In this research, the tests were conducted under various conditions: standard exhaust (Non-DPF), standard exhaust with DPF (DPF), DPF with 50 grams of stainless steel (DPF+SS50), DPF with 100 grams of stainless steel (DPF+SS100), and DPF with 150 grams of stainless steel (DPF+SS150). Based on the test results that have been done, the use of DPF can reduce the exhaust gas opacity of the 1997 Isuzu Panther engine. The 1997 Isuzu Panther engine exhaust gas opacity test results are presented in table 1, figure 5 and figure 6.

Table 1. Exhaust gas opacity test of 1997 Isuzu Panther engine

| Exhaust | Opacity (% HSU) | Opacity Difference (% HSU) | Opacity Reduction (%) |
| --- | --- | --- | --- |
| Non DPF | 90.2 ± 1.1 | - | - |
| DPF | 20.7 ± 3.3 | 69.5 ± 2.6 | 77.0 ± 2.6 |
| DPF+SS50 | 17.5 ± 0.7 | 72.7 ± 0.8 | 80.6 ± 0.8 |
| DPF+SS100 | 15.4 ± 0.5 | 74.8 ± 0.8 | 82.9 ± 0.8 |
| DPF+SS150 | 11.6 ± 0.7 | 78.6 ± 1.1 | 87.1 ± 1.1 |

Figure 5. Exhaust Gas Opacity Test Results

Figure 6. Opacity of the exhaust with DPF and various mass of stainless steel

Smoke Temperature

Based on the test results, the use of DPF can reduce the exhaust temperature of the 1997 Isuzu Panther engine. The test results of the smoke exhaust temperature of the 1997 Isuzu Panther are presented in Table 2, Figure 7 and Figure 8.

Table 2. Smoke Temperature Test of 1997 Isuzu Panther Engine

| Exhaust | Temperature (°C) | Temperature Difference (°C) | Temperature Reduction (%) |
| --- | --- | --- | --- |
| Non DPF | 44 ± 2 | - | - |
| DPF | 37.3 ± 1.5 | 6.7 ± 2.9 | 15.2 ± 2.9 |
| DPF+SS50 | 33.0 ± 1.0 | 11.0 ± 1.7 | 25.0 ± 1.7 |
| DPF+SS100 | 29.0 ± 1.0 | 15.0 ± 1.7 | 34.1 ± 1.7 |
| DPF+SS150 | 25.0 ± 1.0 | 19.0 ± 1.7 | 43.2 ± 1.7 |

Figure 7. Smoke Temperature Test Results

Figure 8. Exhaust smoke temperature with DPF and various mass of stainless steel

Noise Level

Based on the test results, the use of DPF can reduce the noise level of the 1997 Isuzu Panther exhaust. The test results of the 1997 Isuzu Panther noise level are presented in Table 3, Figure 9 and Figure 10.

Table 3. Noise test of 1997 Isuzu Panther engine

| Exhaust | Noise (dB) | Noise Differences (dB) | Noise Reduction (%) |
| --- | --- | --- | --- |
| Non DPF | 102.3 ± 0.9 | - | - |
| DPF | 94.1 ± 0.7 | 8.1 ± 0.7 | 8.0 ± 0.7 |
| DPF+SS50 | 92.2 ± 0.6 | 10.1 ± 0.6 | 9.9 ± 0.6 |
| DPF+SS100 | 89.1 ± 0.6 | 13.2 ± 0.4 | 12.9 ± 0.4 |
| DPF+SS150 | 86.7 ± 0.5 | 15.5 ± 1.3 | 15.2 ± 1.3 |

Figure 9. Noise Level Test Results

Figure 10. Exhaust Noise with DPF and Various Masses of Stainless Steel (SS)

DISCUSSION

Effect Of DPF On Flue Gas Opacity

Figure 5 shows that the standard exhaust produces an exhaust gas opacity level of 90.20% HSU. This result is very large because the standard exhaust does not use DPF as a particulate trap, so the opacity is high [24], [25]. In the standard exhaust, the flow of exhaust gases coming out of the combustion chamber into the free air is not blocked by anything, so that the exhaust gas just comes out without any barriers / traps such as DPF [26]. Referring to the emission quality standards in Indonesia, the exhaust gas opacity of 90.2±1.1%HSU does not meet the emission standard threshold, which in this rule states that the emission quality threshold for diesel vehicles under 2010 is 65% HSU [20].

On experimental exhaust I (DPF) produced an exhaust gas opacity level of 20.7±3.3%HSU. The percentage reduction in exhaust gas opacity was 77.0±2.6%. This result is smaller than the reduction result of the standard exhaust, and it can be said that the opacity decreases. These results show that the use of DPF is very influential on the reduction of diesel engine exhaust opacity [26], [27], [28], [29]. Thus, DPF is very effectively used to reduce exhaust emissions in diesel engine vehicles. Thus, the use of DPF is in accordance with the rules set by EURO VI [30].

Experimental exhaust II (DPF+SS50) produces an exhaust gas opacity level of 17.5±0.7HSU% with a percentage reduction in exhaust gas opacity of 80.6±0.8%. This reduction result is greater than the reduction result of the experimental exhaust I without Stainless steel. Where, the opacity of the resulting exhaust gas has decreased. The amount of Stainless steel added to the DPF is 50 g. This shows that the use of Stainless steel as an additional filter influence reducing the opacity of exhaust gas. The addition of filters proved effective in reducing the opacity of diesel engine exhaust gas.

The exhaust of experiment III (DPF+SS100) produced an exhaust gas opacity level of 15.4±0.5%HSU. The percentage reduction in exhaust gas opacity is 82.9±0.8%. This reduction result is greater than the reduction result of the experimental exhaust II with the addition of Stainless steel 100 gr and it can be said that the opacity decreases. This shows that the addition of more filters can affect the amount of opacity reduced [31].

The experimental exhaust IV (DPF+SS150) produced an exhaust gas opacity level of 11.6±0.7%HSU. The percentage reduction in exhaust gas opacity is 87.1±1.1%. The reduction results are greater than the reduction results of the exhaust experiment I, II, and III. It can be said that the reduction results in the exhaust of experiment IV are the greatest among the four exhaust experiments. The amount of stainless steel added to the DPF is 150 g. These results show that the more filters added in the DPF, the greater the opacity reduction. This indicates that the variation in the amount of stainless steel as a filter added to the DPF is very influential in reducing opacity [32] . The more the number of filters added, the greater the opacity that can be reduced.

The measurement results indicate that the addition of stainless steel mass in the DPF significantly affects exhaust opacity. As shown in Figure 6, there is a linear relationship between the added stainless-steel mass and exhaust opacity. Regression equation obtained in Eq. (1)

| y=-0.0339x+20.128 | (1) |
| --- | --- |

where y represents exhaust opacity and x denote the added stainless steel mass. The coefficient of determination (R²) value of 0.9688 indicates that 96.88% of the variation in exhaust opacity can be explained by the variation in stainless steel mass. From these results, it can be concluded that increasing the mass of stainless steel added to the DPF leads to a decrease in exhaust opacity. This suggests that using heavier stainless steel in the DPF can enhance the filter's efficiency in reducing particulate emissions, thereby contributing to lower air pollution levels.

Effect of DPF on Smoke Temperature

Based on the research data in table 2 and figure 7, it is known that the average value of smoke temperature in the standard exhaust is 44°C, while the experimental exhaust I (DPF) obtained an average temperature of 37.33°C. The addition of stainless steel fiber in DPF shows a lower exhaust gas temperature, namely in the experimental exhaust II (DPF + SS50) the temperature is 33°C, the experimental exhaust III (DPF + SS100) the temperature is 29°C, and the experimental exhaust IV (DPF + SS150) the temperature is 25°C.

Experimental exhaust I (DPF) produced a smoke temperature of 37.3±1.5°C. The percentage reduction in exhaust gas opacity was 15.2±2.9%. This result is smaller than the reduction result of the standard exhaust, and it can be said that the temperature decreases. These results show that the use of DPF is very influential in reducing the smoke temperature of diesel engines. Thus, DPF is very effectively used to reduce smoke temperature in diesel engine vehicles.

The experimental exhaust II (DPF+SS50) produced a smoke temperature of 33±1°C with a percentage reduction in exhaust gas opacity of 25±1.7%. These reduction results are greater than the reduction results of the experimental exhaust I without Stainless steel. Where, the smoke temperature produced has decreased. The amount of stainless steel added to the DPF is 50 g. This shows that the use of stainless steel as an additional filter has an effect in reducing the smoke temperature. The addition of filters has proven effective in reducing the smoke temperature of diesel engines.

The experimental exhaust III (DPF+SS100) produced a smoke temperature level of 29±1°C. The percentage reduction in exhaust gas opacity was 34.1 ± 1.7%. These reduction results are greater than the reduction results of the experimental exhaust II with the addition of Stainless steel 100 gr and can be said to have decreased temperature. This shows that the addition of filters proved effective in reducing the smoke temperature of diesel engines.

Exhaust experiment IV (DPF+SS150) produced a smoke temperature level of 25±1°C. The percentage of smoke temperature reduction was 43.2±1.7%. The reduction results are greater than the reduction results in the exhaust experiments I, II, and III. It can be said that the reduction results in the exhaust experiment IV are the greatest among the four exhaust experiments. The amount of stainless steel added to the DPF is 150 g. This result shows that the more filters added to the DPF, the greater the temperature reduction.This indicates that the variation in the amount of stainless steel as a filter added to the DPF is very influential in reducing temperature. The more the number of filters added, the greater the temperature that can be reduced. The measurement results show that the addition of steinless steel fiber to the DPF significantly affects the smoke temperature. As shown in Figure 8, there is a linear relationship between the added steinless steel fiber and the smoke temperature. Regression equation obtained in Eq. (2)

| y=-0.0471x + 36.384 | (2) |
| --- | --- |

Where y indicates the smoke temperature and x indicates the added steinless steel fiber. The coefficient of determination (R²) value of 0.9737 indicates that 97.37% of the variation in smoke temperature can be explained by the variation in steinless steel fiber. This study does not specifically measure the temperature during the regeneration process, but discusses the difference in smoke temperature in the exhaust of diesel vehicles installed with DPF and standard exhaust. The results showed that the addition of stainless steel (SS) fibers in the DPF can significantly reduce the smoke temperature. The exhaust gas flow in the DPF is blocked by trapped particles, so this can cause a decrease in smoke temperature. The decrease in temperature in some DPF configurations with SS fibers indicates a more efficient heat transfer mechanism.

The decrease in average temperature from 44°C (standard exhaust) to 37.33°C (experimental exhaust I) demonstrates the ability of copper to absorb and distribute heat more effectively. The high thermal conductivity of copper (390 W/m.K at room temperature) enables rapid heat transfer from the exhaust gas to the DPF substrate, thereby reducing the temperature of the exiting smoke [33]. The addition of SS fibers showed a more significant reduction in smoke temperature, reaching 43.2% in the DPF+SS150 configuration (average temperature 25°C). Despite having a lower thermal conductivity than copper, the presence of stainless steel serves as an additional heat spreading material. These fibers increase the surface area for convective heat transfer, accelerating heat release from the flue gas to the filter substrate. This theory is in line with research on porous materials for heat transfer applications, where a larger surface area increases heat transfer efficiency [34].

The greater temperature reduction with increased stainless steel fiber density (DPF+SS50, DPF+SS150, DPF+SS150) can be explained by conduction and convection heat transfer theories. Conduction theory suggests that the addition of stainless steel fibers increases the heat transfer path in the substrate, which accelerates the release of thermal energy from the flue gas, and Convection theory suggests that the flue gas comes into contact with more surfaces of the heat-absorbing material, increasing the heat transfer from the gas to the substrate [35]. The temperature reduction achieved shows great potential in reducing heat emissions to the environment and improving thermal efficiency. This result is consistent with previous findings showing that the integration of heat-absorbing materials into the DPF substrate can improve thermal efficiency and significantly reduce smoke temperature [36]. Thus, the addition of stainless steel fibers in the copper-based DPF significantly improved the ability of the DPF to absorb and disperse heat, and reduced the smoke temperature by 43.2%.

Effect of DPF on Noise Level

Figure 9 shows that the standard exhaust produces a noise level of 102.3±0.9 dB. This result is very large because the standard exhaust does not use DPF as a silencer so the noise is high. There are no additional components in the standard exhaust that filter solid particles from the exhaust gas before it is released into the atmosphere. Referring to the vehicle noise quality standards in Indonesia, the noise of 102.3 ± 0.9 dB has not met the established threshold [37].

Experimental exhaust I (DPF) produced a noise level of 94.1 ± 0.7 dB. The percentage of noise reduction was 8±0.7%. This result is smaller than the result of the standard exhaust noise, and it can be said that the noise decreases. These results show that the use of DPF is very influential in reducing diesel engine noise [38].

The use of a copper-based DPF in a diesel exhaust system can reduce noise because the DPF can work to filter out fine particles from the exhaust gas, which not only reduces particle emissions but also contributes to noise reduction. This filtering process changes the flow characteristics of the exhaust gas, which can reduce the sound produced by the diesel engine [38]. In the filtration process, the honeycomb structure of the DPF in this study causes turbulence and a decrease in sound wave energy, which helps to reduce exhaust noise. In addition, the copper material can serve as a catalyst in the oxidation process, which helps reduce harmful gas emissions and improve combustion efficiency, thereby contributing to noise reduction [39].

Exhaust experiment II (DPF+SS50) produced a noise level of 92.2 ± 0.6 dB with a noise reduction percentage of 9.9 ± 0.6%. The results of this decrease are greater than the results of the decrease in noise from Exhaust I without Stainless steel. Where, the noise produced has decreased. The amount of stainless steel added to the DPF is 50 gr. This shows that the use of stainless steel as an additional filter influences noise reduction [40]. The addition of filters is proven to be effective in reducing diesel engine noise.

The third experimental exhaust (DPF+SS100) produced a noise level of 89.1±0.6 dB. The percentage of noise reduction is 12.9 ± 0.4%. This reduction result is greater than the reduction result of the experimental exhaust II, with the addition of 100 gr stainless steel and it can be said that the noise has decreased. This shows that the addition of more filters can affect the amount of noise reduced [40].

Exhaust experiment IV (DPF+SS150) produced a noise level of 86.7 ± 0.5 dB. The percentage of noise reduction was 15.2±1.3%. The reduction results are greater than the reduction results of the exhaust experiment I, II, and III. It can be said that the exhaust gas reduction results of experiment IV are the greatest among the four exhaust experiments. The amount of stainless steel added to the DPF is 150 gr. This result shows that the more filters added to the DPF, the greater the noise reduction. This shows that the variation in the amount of stainless steel as a filter added to the DPF is very influential in reducing noise [38], [40]The more the number of filters added, the greater the noise that can be reduced.

The measurement results show that the addition of steinless steel fiber to the DPF significantly affects the exhaust noise. As shown in Figure 10, there is a linear relationship between the added steinless steel fiber and the noise. Regression equation obtained in Eq. (3)

| y=-0.0471x + 36.384 | (3) |
| --- | --- |

Where y indicates the noise and x indicates the added steinless steel fiber. The coefficient of determination (R²) value of 0.9901 indicates that 99.01% of the noise variation can be explained by the variation of steinless steel fiber.

From these results, it can be concluded that increasing the steinless steel fiber added to the DPF leads to a decrease in noise. This shows that the use of heavier steinless steel in the DPF can increase the efficiency of the filter in reducing noise. When stainless fibers are added, the filter structure becomes more efficient in capturing particles, which not only increases the filtration efficiency but also reduces the noise generated by the exhaust gas flow [41]. The use of copper based DPFs and the addition of stainless fibers in DPFs make a significant contribution to noise reduction in diesel exhaust systems, although in this study only Experiment IV exhaust (DPF+SS150) has met the noise standards of diesel car vehicles in Indonesia, which is below 87 dB [37].

CONCLUSION

# This research shows that DPF significantly affects the reduction of exhaust opacity, smoke temperature and noise level of 1997 Isuzu Panther. The largest reduction in exhaust opacity, smoke temperature and noise level is produced by DPF + 150 gr stainless steel with a reducible exhaust opacity of 87.1 ± 1.1%, smoke temperature of 43.2 ± 1.7%, and noise level of 15.2 ± 1.3%.

# ACKNOWLEDGMENT

Our gratitude goes to the Ministry of Education, Culture, Research, and Technology of the Republic of Indonesia for providing full financial support for the Regular Fundamental Research Grant (PFR) with contract number 075/E5/PG.02.00.PL/2024. We also appreciate the University of Palangka Raya for providing support and facilitating us in this research.

REFERENCES

[1]	S. Indonesia, “Number of Motorized Vehicles by Province and Vehicle Type (units), 2023,” Statistics Indonesia. Accessed: Nov. 08, 2024. [Online]. Available: https://www.bps.go.id

[2]	I. N. P. T. Corps, “Total Vehicle Data Per Regional Police,” http://rc.korlantas.polri.go.id. Accessed: Jan. 07, 2025. [Online]. Available: http://rc.korlantas.polri.go.id:8900/eri2017/laprekappolda.php

[3]	G. Sharmilaa and T. Ilango, “Vehicular air pollution based on traffic density - A case study,” Mater Today Proc, vol. 52, pp. 532–536, 2022, doi: https://doi.org/10.1016/j.matpr.2021.09.278.

[4]	G. Samuels and Y. Freemark, “The Polluted Life Near the Highway A Review of National Scholarship and a Louisville Case Study,” 2022.

[5]	F. An, J. Liu, W. Lu, and D. Jareemit, “A review of the effect of traffic-related air pollution around schools on student health and its mitigation,” J Transp Health, vol. 23, p. 101249, 2021, doi: https://doi.org/10.1016/j.jth.2021.101249.

[6]	L. H. Watkins, Air pollution from road vehicles, 1st ed. London: HMSO, 1991. Accessed: Nov. 12, 2024. [Online]. Available: https://trl.co.uk/uploads/trl/documents/SAR-01---Air-pollution-from-road-vehicles.pdf

[7]	S. Saarikoski et al., “Towards zero pollution vehicles by advanced fuels and exhaust aftertreatment technologies,” Environmental Pollution, vol. 347, p. 123665, 2024, doi: https://doi.org/10.1016/j.envpol.2024.123665.

[8]	V. K. Lyubov, A. N. Popov, and E. I. Popova, “Emission of soot particles from the combustion of various fuels in boilers,” in IOP Conference Series: Earth and Environmental Science, IOP Publishing Ltd, Oct. 2021. doi: 10.1088/1755-1315/866/1/012010.

[9]	C. A. Weitekamp, L. B. Kerr, L. Dishaw, J. Nichols, M. Lein, and M. J. Stewart, “A systematic review of the health effects associated with the inhalation of particle-filtered and whole diesel exhaust,” Jan. 02, 2020, Taylor and Francis Ltd. doi: 10.1080/08958378.2020.1725187.

[10]	J. Hu, R. Zhou, R. Ding, D.-W. Ye, and Y. Su, “Effect of PM2.5 air pollution on the global burden of lower respiratory infections, 1990–2019: A systematic analysis from the Global Burden of Disease Study 2019,” J Hazard Mater, vol. 459, p. 132215, 2023, doi: https://doi.org/10.1016/j.jhazmat.2023.132215.

[11]	P. Thangavel, D. Park, and Y. C. Lee, “Recent Insights into Particulate Matter (PM2.5)-Mediated Toxicity in Humans: An Overview,” Jun. 01, 2022, MDPI. doi: 10.3390/ijerph19127511.

[12]	Z. Zhang, R. Dong, G. Lan, T. Yuan, and D. Tan, “Diesel particulate filter regeneration mechanism of modern automobile engines and methods of reducing PM emissions: a review,” Mar. 01, 2023, Springer Science and Business Media Deutschland GmbH. doi: 10.1007/s11356-023-25579-4.

[13]	S. Su, T. Lv, Y. Lai, J. Mu, Y. Ge, and B. Giechaskiel, “Particulate emissions of heavy duty diesel engines measured from the tailpipe and the dilution tunnel,” J Aerosol Sci, vol. 156, p. 105799, 2021, doi: https://doi.org/10.1016/j.jaerosci.2021.105799.

[14]	M. Z. Mon Phyo et al., “Experimental investigation of metallic partial-flow particulate filter on a diesel engine’s combustion pressure and particle emission,” Case Studies in Thermal Engineering, vol. 49, Sep. 2023, doi: 10.1016/j.csite.2023.103188.

[15]	D. Bakhchin, R. Ravi, M. Faqir, and E. Essadiqi, “A technical review on low temperature combustion alternatives for ultra-low emission vehicles,” Journal of the Energy Institute, vol. 111, p. 101410, 2023, doi: https://doi.org/10.1016/j.joei.2023.101410.

[16]	J. W. Cresswell and J. D. Cresswell, Research Design: Qualitative, Quantitative, and Mixed Methods Approaches. Fifth Edition, vol. 5. 2018.

[17]	S. Rizki Ariyanto, M. Yandi Pratama, and K. Refa Haratama, “Optimization of Metallic Catalytic Converters to Reduce CO Emissions and Increase Engine Power,” vol. 7, no. 2, pp. 299–309, 2024, doi: 10.31603/ae.11567.

[18]	M. L. Godoy, V. G. Milt, E. E. Miró, and E. D. Banús, “Scaling-up of the catalytic stacked wire mesh filters for the abatement of diesel soot,” Catal Today, vol. 394–396, pp. 434–444, 2022, doi: https://doi.org/10.1016/j.cattod.2021.07.010.

[19]	H. P. Buwono et al., “Performance of Transition Metal Supported Al2O3 Coated on Honeycomb Catalysts and Its Segmentation on Exhaust Gasses Oxidation,” Automotive Experiences, vol. 7, no. 1, pp. 86–96, Jan. 2024, doi: 10.31603/ae.10686.

[20]	M. of E. and F. of the R. of Indonesia, “Regulation of the Minister of Environment and Forestry of the Republic of Indonesia Number 8 of 2023,” Jakarta, 2023. Accessed: Nov. 12, 2024. [Online]. Available: https://jdih.menlhk.go.id

[21]	M. of T. of the R. of Indonesia, “Regulation of the Minister of Transportation of the Republic of Indonesia Number PM 23 of 2021 concerning the Second Amendment to the Regulation of the Minister of Transportation Number PM 33 of 2018 concerning Motor Vehicle Type Testing,” Jakarta, 2021. Accessed: Nov. 12, 2024. [Online]. Available: https://jdih.kemenhub.go.id

[22]	Unknown, “Society of Automotive Engineers (SAE) J1667 Recommended Practice Snap Acceleration Smoke Test Procedure for Heavy-Duty Powered Vehicles,” 1998. [Online]. Available: http://www.sae.org

[23]	“ISO/FDIS 5130 Acoustics-Measurements of sound pressure level emitted by stationary road vehicles Acoustique-Mesurages du niveau de pression acoustique émis par les véhicules routiers en stationnement,” 2006. [Online]. Available: www.iso.org

[24]	S. Drašković, D. Vukić, V. Petrović, and G. Dikić, “COMPARISON OF PM PRODUCTION IN DIESEL ENGINE EXHAUST GASES: WITH AND WITHOUT DIESEL PARTICULATE FILTER,” Serbia, Aug. 2021. Accessed: Nov. 12, 2024. [Online]. Available: https://engineering-conference.rs/EC2023/Engineering%20Conference%202023%20Budva/zbornik_radova_Budva2023.html

[25]	J. B. Heywood, Internal Combustion Engine Fundamentals, 2nd Edition. New York: McGraw-Hill Education, 2018. [Online]. Available: https://www.accessengineeringlibrary.com/content/book/9781260116106

[26]	G. Park, S. Park, T. Hwang, S. Oh, and S. Lee, “A Study on the Impact of DPF Failure on Diesel Vehicles Emissions of Particulate Matter,” Applied Sciences (Switzerland), vol. 13, no. 13, Jul. 2023, doi: 10.3390/app13137592.

[27]	G. Nain Singh and R. Singh Bharj, “Experimental study of filtration behavior of diesel particulate filter in a diesel engine to meet BS-VI emission norms in INDIA,” in Journal of Physics: Conference Series, Institute of Physics Publishing, Aug. 2019. doi: 10.1088/1742-6596/1276/1/012078.

[28]	G. Wu et al., “A Review of Thermal Energy Management of Diesel Exhaust after-Treatment Systems Technology and Efficiency Enhancement Approaches,” Feb. 01, 2024, Multidisciplinary Digital Publishing Institute (MDPI). doi: 10.3390/en17030584.

[29]	H. Pranoto, D. Feriyanto, and S. Zakaria, “PERFORMANCE AND EXHAUST GAS TEMPERATURE INVESTIGATION OF CERAMIC, METALLIC AND FeCrAl CATALYTIC CONVERTER IN GASOLINE ENGINE,” SINERGI, vol. 23, no. 1, p. 11, Feb. 2019, doi: 10.22441/sinergi.2019.1.002.

[30]	M. Williams and R. Minjares, “A technical summary of Euro 6/VI vehicle emission standards,” 2016. [Online]. Available: www.theicct.org

[31]	R. Almahdi, A. Khalil, A. S. Eshtiwi, and A. A. Tantoush, “The Role of The Carbon Filter (Catalyst) in Cars and The Effect of Its Removal,” Acta Mechanica Malaysia, vol. 7, no. 1, pp. 23–27, 2024, doi: 10.26480/amm.01.2024.23.27.

[32]	S. Khasim Sharif and J. Nagarjun, “Design and Modelling of Diesel Particulate Filter to Reduce the Emissions from CI Engine,” International Journal of Mechanical Engineering and Technology (IJMET), vol. 10, no. 1, pp. 2133–2140, 2019.

[33]	A. M. Abyzov, F. M. Shakhov, A. I. Averkin, and V. I. Nikolaev, “Mechanical properties of a diamond–copper composite with high thermal conductivity,” Mater Des, vol. 87, pp. 527–539, Dec. 2015, doi: 10.1016/j.matdes.2015.08.048.

[34]	M. Xu, C. Shi, P. Li, X. Guo, B. Wang, and D. Zou, “Thermal Expansion Challenges and Solution Strategies for Phase Change Material Encapsulation: A Comprehensive Review,” Adv Funct Mater, vol. 34, no. 51, Dec. 2024, doi: 10.1002/adfm.202409884.

[35]	N. A. S. Muzaidi et al., “Heat absorption properties of CuO/TiO2/SiO2 trihybrid nanofluids and its potential future direction towards solar thermal applications,” Arabian Journal of Chemistry, vol. 14, no. 4, p. 103059, 2021, doi: https://doi.org/10.1016/j.arabjc.2021.103059.

[36]	Z. Meng et al., “Experimental study on regeneration performance and particle emission characteristics of DPF with different inlet transition sections lengths,” Fuel, vol. 262, p. 116487, 2020, doi: https://doi.org/10.1016/j.fuel.2019.116487.

[37]	R. Witoelar, “Regulation Of The Minister Of Environment Number 07 Of 2009 Concerning The Noise Threshold Of New Type Motor Vehicles,” Jakarta, 2009. Accessed: Jan. 09, 2025. [Online]. Available: https://ppkl.menlhk.go.id/website/filebox/751/190930173417permen_lh_07_2009_ambang_batas_kebisingan_kendaraan_baru.pdf

[38]	T. Yamamoto, Y. Akimoto, and N. Hosomi, “Multiscale simulation for sound transmission loss of a particulate filter in an exhaust system using a homogenization method,” Applied Acoustics, vol. 219, p. 109939, 2024, doi: https://doi.org/10.1016/j.apacoust.2024.109939.

[39]	H. Papadopoulou, S. Papagianni, A. M. Moschovi, K. M. Sakkas, and I. Yakoumis, “DES-Based Solution for Regenerating Diesel Particulate Filters of Euro V/VI Diesel Vehicles †,” in International Conference on Raw Materials and Circular Economy, Basel Switzerland: MDPI, Jan. 2022, p. 107. doi: 10.3390/materproc2021005107.

[40]	J. K. Lee, K. S. Oh, and J. W. Lee, “Methods for evaluating in-duct noise attenuation performance in a muffler design problem,” J Sound Vib, vol. 464, p. 114982, 2020, doi: https://doi.org/10.1016/j.jsv.2019.114982.

[41]	S. M. Fayyad, “Controlling Noise and Gas Emissions by a New Design of Diesel Particulate Filters,” WSEAS TRANSACTIONS ON APPLIED AND THEORETICAL MECHANICS, vol. 15, pp. 69–81, Jun. 2020, doi: 10.37394/232011.2020.15.10.